/* 
	C program to simulate receiver side logic to decipher:
	1. Frame is valid or not
	2. Type of frame (I-frame, S-frame, U-frame)
	3. Data
	4. Checksum bits  ( Assuming only 2-byte)
	5. code bits
	6. Sequence numbers
	7. P/F bit values
	8. Addresses, single byte or multi-byte
*/

---------------------------------------------------------------------------------------------------------

#include<stdio.h>

#include<malloc.h>

// If any of the values of valid array becomes 0, the data is valid

static int valid[3]={1,1,1}; 	//Flag1, Address, Flag2

static int vData = 1; 	//Data will be present for I-frame and U-frame

/* IDENTIFYING THE ADDRESS TYPE */
int Address(int bit[],int len)

{
	/*  As the address ends with bit 1 at the last position in case of both single-byte and multi-byte addresses, we check for the last bit position excluding the flag bits (8)+first 7 bits of the address field(7)=14 i.e;(0-14)bit positions. */

int i=15,c=1;

while(bit[i] == 0 && i<len)

{

i+=8;

c++;

}

if(bit[i] == 0 && !(i<len)) 		// Invalid Address means it shouldn't end with 1

valid[1]=0;

return c;

}

/* IDENTIFYING THE TYPE OF FRAME */

char Frame(int bit[],int bytes)

{
	/* The structures of S,I and U-frames vary based on the bit positions in the control field as :  Frame                Control Field
			  ( 0 1 2 3 4 5 6 7 )
	   I-frame  :           0 ..................
	   S-frame  :          1 0 ...............
	   U-frame  :         1 1 ............... */

int p = 8*(bytes+1); 	/* Since the control field begins at the end of the 						     Address field.*/
	if(bit[p] == 0)

return 'I';

else if(bit[p] == 1 && bit[p+1] == 0)

return 'S';

else

return 'U';

}

/* IDENTIFYING DATA BITS IN THE BIT STREAM */

int Data(int bit[],char fr,int bytes,int len)

{
	/* If the frame is S-frame, then there is no data.
 	     If the frame is I-frame, User information exists.
  	     If the frame is U-frame, System information exists.
 	     If any kind of information exists, it begins at the end of control field.	*/

int i=0,p = bytes+2,q = 8*p;

if(fr == 'S')

{

if (len > (8*p)+24 ) 	/*    8 * (start_flag + control + address bytes) + 							cksum(16) + end_flag   */

{

vData = 0;

return i;

}

else

printf("\nNo Data Bits");

}

else if(fr == 'I')

{

if (len <= (8*p)+24 ) 	/*    8 * (start_flag + control + address bytes) + 							cksum(16) + end_flag   */

{

vData = 0;

return i;

}

printf("\nUser Information is:");

for(i=q;i<len-24;i++)

printf("%d",bit[i]);

}

else

{

if (len <= (8*p)+24 ) 	/*    8 * (start_flag + control + address bytes) + 							cksum(16) + end_flag   */

{

vData = 0;

return i;

}

printf("\nSystem Information is:");

for(i=q;i<len-24;i++)

printf("%d",bit[i]);

}

return i;

}

/* IDENTIFYING CHECKSUM */

void CkSum(int bit[],char fr,int bytes,int len,int d)

{
	/*  Assuming that the Checksum is only 2-byte long, we make put the condition that 	    the Checksum ends at totalLength-8.
	    We check the same for S,I,U-frames.   */

int i,p = bytes+2;

printf("\nChecksum Bits: ");

if(fr=='S')

{

for(i=8*p;i<len-8;i++)

printf("%d",bit[i]);

}

else

for(i=d;i<len-8;i++)

printf("%d",bit[i]);

}

/* IDENTIFYING CODE BITS */
void Code(int bit[],char fr,int bytes)

{
	/* If the frame is I-frame, no code bits will be present.
	   Otherwise, the code bits are:

Frame		      CODE BITS in Control Field
 					(  0 1 2 3 4 5 6 7  )
	   S-frame:              		       - -
	   U-frame:			       - -   - - -
	 */

int p = 8*(bytes+1);

if(fr == 'I')

printf("\nNo Code Bits");

else if(fr == 'S')

printf("\nCode Bits are: %d %d",bit[p+2],bit[p+3]);

else

printf("\nCode Bits are: %d %d %d %d %d",bit[p+2],bit[p+3],bit[p+5],bit[p+6],bit[p+7]);

}

/* IDENTIFYING P/F BIT */

void PFval(int bit[],int bytes,char fr)

{
	/*The P/F bit in all the three frames is:
	  Frame             P/F in Control Field..
	  	   	  (  0 1 2 3 4 5 6 7  )
	  S (or) I (or) U:                   -
	*/

int p = 8*(bytes+1);

printf("\nP/F Value = %d",bit[p+4]);

}

/* IDENTIFYING SEQUENCE NUMBER */
void Seq(int bit[],char fr,int bytes)

{
	/* If the frame is S-frame or U-frame, Sequence bits are absent.
	     If the frame id I-frame, the position of Sequence bits are given by:
                  Bit Positions in Control Field:  0 1 2 3 4 5 6 7
					          - - -
	*/
	int p=8*(bytes+1);

if(fr == 'S' || fr == 'U')

printf("\nNo Sequence Number");

else

printf("\nSequence Number (Binary format) is: %d%d%d",bit[p+1],bit[p+2],bit[p+3]);

}

/* CHECKING THE VALIDITY OF THE FRAME */
 /* A frame is invalid when:
    1. S-Frame contains data
    2. I or U-Frame doesn't contain data
    3. Either a single byte or a multi bytes address doesn't terminate with a 1
    4. When the flag bits are not properly placed
    5. When Checksum at sender and receiver doesn't match
    (However, we doesn't check the checksums here because we don't know the generator as well as checksum at sender side) 
 */

int Valid(int *bit)

{

if(valid[0] == 0 || valid[1] == 0 || valid[2] == 0 || vData == 0)

{

printf("\nGIVEN FRAME IS INVALID");

return 0;

}

else

{

printf("\nGIVEN FRAME IS VALID");

return 1;

}

}

/* CHECKING START_FLAG */

void Flag1(int *bit)

{

int i=0;

if(bit[0] != 0 || bit[7] != 0)

valid[0]=0;

for(i=1;i<7;i++)

if(bit[i] != 1)

{

valid[0]=0;

break;

}

}

/* CHECKING END_FLAG */

  void Flag2(int *bit,int len)

{

int i=len;

if(bit[len-1] != 0 || bit[len-8] != 0)

valid[2]=0;

for(i=len-7;i<len-1;i++)

if(bit[i] != 1)

{

valid[2]=0;

break;

}

}

/* MAIN BEGINS */

int main()

{

int len,bytes,i,d,*bit,validity=0;

char fr;

//Assuming that the receiver received the following bit stream

printf("\nEnter the length of the Bit Stream received: ");

scanf("%d",&len);

bit=(int *)malloc(len * sizeof(int));

printf("\nEnter the Bit Stream:\n");

for(i=0;i<len;i++)

scanf("%d",&bit[i]);

Flag1(bit);

Flag2(bit,len);

bytes = Address(bit,len);		//returns the type of address to 'bytes'

	fr = Frame(bit,bytes);		//returns a character indicating frame type

	d = Data(bit,fr,bytes,len);

validity = Valid(bit);

if(validity != 0)

{

printf("\nThe frame is: %c",fr);		//displays data if any

CkSum(bit,fr,bytes,len,d);	//prints checksum in binary format

Code(bit,fr,bytes);		//prints code bits if any

Seq(bit,fr,bytes);		//prints sequence number if any

PFval(bit,bytes,fr);		//prints P/F bit

printf("\n%d byte Address",bytes);

}

return 0;

}

---------------------------------------------------------------------------------------------------------

OUTPUT: